# SPECIAL SIGNING “FRESH OIL AHEAD”

Revised on: 4/6/2016

A minimum of 24 hours prior to applying Bituminous Materials (Tack Coat) and Aggregate (Tack Coat), the Contractor shall install one sign approximately 500 feet in advance of each end of the area to be tacked.  Additional signs shall be installed for all side roads within the area to be tacked.  If the area to be tacked does not begin at the intersection of two roads an additional sign shall be installed at the nearest intersection preceding the area to be tacked.  These signs shall remain in place until the Leveling Binder course has been placed or until the Tack Coat has fully cured as determined by the Engineer.

These signs shall be clean and legible and have minimum dimensions of 48 inches by 48 inches and have a black legend and border on an orange reflectorized background.  The legend shall read:

“FRESH OIL AHEAD”

“15 M.P.H.”

The installation and the lettering shall conform to the MUTCD and meet the approval of the Engineer.

Basis of Payment: This work shall not be paid for separately but shall be considered incidental to the pay item BITUMINOUS MATERIALS (TACK COAT), which shall include all labor, materials, and equipment necessary to complete the work described above.