# FULL-ACTUATED CONTROLLER AND CABINET (KDOT Supplement)

Updated: 10/1/2025

Revise IDOT D1 special provision of FULL-ACTUATED CONTROLLER AND CABINET to add the following:

Materials.

The controller firmware version shall be compatible with KDOT ATMS, TransSuite.

Controller cabinet shall come installed with a sidewall interior mounted power strip with additional Ethernet/IP functionality detailed later in specification.

Add the following to Article 1074.03

(b) (1) (f)	Malfunction Management Unit shall be have a Network interface card (NIC) and associated RJ45 port so that device can be communicative over an Ethernet (fiber optic) network.

(b) (1) (g)	Malfunction Management Unit (Make/model/firmware) shall natively support flashing yellow arrow monitoring capability.

(b) (22)	Power Strip, shall have a Network Interface through RJ 45 port Ethernet communications. The power switch shall have a minimum of 8 outlets which are remotely switched and 2 outlets which are always on. Shall also support functionality for automatically pinging IP addresses with a programmable function to reboot user designated outlets.