# REMOTE CONTROLLED VIDEO SYSTEM (KDOT SUPPLEMENT)

Updated: 10/1/2025

Revise IDOT D1 special provision of REMOTE CONTROLLED VIDEO SYSTEM to add the following:

Materials.

The PTZ camera shall be one of the following approved models or equal to.

- AXIS Q6135-LE

- AXIS Q6355-LE

- AXIS Q6358-LE

The camera shall meet the following environmental and operational conditions:

- A minimum operating temperature range of –50 °C to +55 °C (–58 °F to +131 °F) when powered with a 60 W midspan.

- A maximum temperature tolerance of up to 74 °C (165 °F), in accordance with NEMA TS 2, Section 2.2.7.

- Arctic Temperature Control functionality, enabling start-up at temperatures as low as –40 °C (–40 °F).

- Operation in humidity conditions from 10% to 100% relative humidity (RH), including condensing environments.

- Ruggedized outdoor design that complies with IP66, IK10, and NEMA 4X ratings.

The camera shall support:

- Zipstream H.264 and H.265 video compression technology for efficient bandwidth usage.

- Electronic Image Stabilization (EIS) to reduce vibration-related blur.

- Software-based defogging technology.

- OptimizedIR to enhance visibility in low-light and nighttime environments.